[image: image1.jpg]



[image: image2.png]



[image: image3.png]B | i e ExPORT

A

amate audio

Great sound
from Barcelona
since 1972

Amate Audio S.L.

EXPORT & CUSTOMER SERVICE
Perpinya, 25 - Poligon Industrial Nord
08226 Terrassa - Barcelona - SPAIN
T1.+3493 735656
export@amateaudio.com

www.amateaudio.com

R&D, FACTORY AND MANAGEMENT
Violinista Vellsola, 18

08222 Terrassa - Barcelona - SPAIN
T. 43493 736 23 90
info@amateaudio.com





[image: image5.png]www.amateaudio.com

A

amate audio
Great sound
from Barcelona
since 1972
Amate Audio S.L. EXPORT & CUSTOMER SERVICE R&D, FACTORY AND MANAGEMENT
Perpinya, 25 - Poligon Industrial Nord Violinista Vellsola, 18
08226 Terrassa - Barcelona - SPAIN 08222 Terrassa - Barcelona - SPAIN
T. 43493 7356565 T. 43493 736 23 90

export@amateaudio.com info@amateaudio.com




[image: image4.png]www.amateaudio.com

A

amate audio

Great sound
from Barcelona
since 1972





Architectural and engineering specifications

S18W 

• The system shall consist of an active subwoofer enclosure. The enclosure shall feature a direct radiating, 4-inch voice coil, neodymium magnet, 18-inch loudspeaker component, loaded in a band-reflex tuned enclosure.  Low frequency transducer shall be rated at a nominal impedance of 4 ohm.
• The usable bandwidth shall be 33Hz to 140Hz (-10dB). 

• Maximum SPL continuous, measured at 1m under free field conditions shall be 132 dB. Maximum SPL peak, measured at 1m under free field conditions shall be 135 dB.

• Enclosure shall be band-reflex type with lateral laminar ports.

• Enclosure cabinet construction shall consist of birch plywood. Dimensions shall be 740 mm (29,1 inch) wide, 555 mm (21,8 inch) high, 748 mm (29,4 inch) deep. Enclosure weight shall be 56 Kg. 
• Enclosure cabinet finish shall consist of black matt high-resistance Polyurea® paint.

• The front enclosure shall be protected by two 2mm steel grilles with acoustically transparent black technical mesh. The grille shall be secured via screws to keep it in place when facing the cabinet downwards.

• The cabinet shall feature a round socket with M20 thread located on the top panel to allow full-range enclosure coupling. 
• The cabinet shall feature several M6 rigging points to fix an external rigging hardware.  

• The amplifier shall be built inside the cabinet, consisting of one channel, with a total power of 2500W/4Ohm. The amplifier shall be 3rd Generation Class D.

• The amplifier’s power supply shall be switching mode equipped with Power Factor Correction and accept a universal input voltage (85 to 265 VAC). The power supply shall protect itself in case of excessive voltage at the mains and rearm itself when the voltage is in the specified range. Mains supply shall be through PowerCon, and a PowerCon Link connector shall be available. 
 
• The analogue audio input shall be electronically balanced, with a minimum input impedance of 50kOhm. Connectors shall be XLR type (one female for input and one male for linking). The nominal input shall be of +2dBu, and the input shall accept up to +20dBu without distortion. 

• The system shall be equipped with an in-built digital signal processor (DSP), integrated to the power amplifier. The internal DSP architecture shall be 64-bit double precision, featuring IIR and FIR filters. Analogue-to-Digital converters shall have a signal-to-noise ratio better than 98dB, 24-bit word size and shall work at 48 kHz sampling rate. 

• The DSP shall contain factory and the possibility of user-defined presets. The presets and the main DSP parameters such as gain, limiter threshold and delay shall be visible and selectable by the user at the cabinet’s rear panel via a built-in 3.5” TFT touchscreen. The available memory for the delay line shall allow up to 100m (294 milliseconds).

• The DSP board shall be equipped with an internal USB port for firmware update and maintenance purposes. The port shall be accessed by removing the amplifier from the cabinet. The connector used will be USB-C.

• The loudspeaker system shall be the Amate Audio® NITID S18W.

• The rigging accessories shall be the Amate Audio® KRW118 and BRWX.

September 2025
